UNITED STATES OF AMERICA

# DRAFT PRELIMINARY VIEWS FOR WRC-15

Agenda Item 1.13:  to review No. 5.268 with a view to examining the possibility for increasing the 5 km distance limitation and allowing space research service (space-to-space) use for proximity operations by space vehicles communicating with an orbiting manned space vehicle, in accordance with Resolution 652  (WRC 12)

BACKGROUND:  WARC-92 allocated the band 410-420 MHz to the space research service (SRS) on a secondary basis to allow for extra-vehicular activity (EVA) communications in the immediate vicinity of low earth orbit (LEO) manned space vehicles.  EVA refers to manned activities outside a spacecraft (e.g., spacewalk).  No. 651A (WARC-92) limited the use of the band by the SRS to EVA operation within 5 kilometers (km) of orbiting manned space vehicles.  WRC-97 upgraded the allocation to the SRS in the band 410-420 MHz to primary status and No. 5.268 specified a set of power flux-density (pfd) limits to ensure protection of the fixed and mobile services while retaining the 5 km distance limitation for EVA operation.

Resolution 652 (WRC-12), recognizing c, states that “power flux-density (pfd) limits contained in No. 5.268 ensure the protection of terrestrial stations operating in the fixed and mobile services independent of the distance from, or the source of, space-to-space communications in the SRS.”  ITU-R preliminary analyses using a spread spectrum signal in the 410-420 MHz band by a LEO vehicle suggest that these vehicle links can meet the pfd limits in No. 5.268 for distances beyond 5 km.  Long-term space exploration objectives require new activities around a manned space station other than EVA, such as visiting vehicles for crew transportation/cargo re-supply and free-fly proximity vehicles for inspection and maintenance.  These vehicles need to initiate communication over distances greater than 5 km to ensure proper vehicle positioning, data exchange and system monitoring.  Therefore, it is necessary to modify No. 5.268 to remove the 5 km distance restriction and EVA limitation while maintaining the pfd limits.

U.S. VIEW:  The United States supports the removal of both the 5 km distance limitation and restriction to EVA operation if the studies, in accordance with Resolution 652 (WRC-12), demonstrate space vehicle links operating around a manned vehicle beyond 5 km can meet the pfd limits in No. 5.268.  Removal of these two restrictions will allow for greater flexibility in using the band 410-420 MHz for space research activities while maintaining protection of the terrestrial services.