# UNITED STATES OF AMERICA

# DRAFT PRELIMINARY VIEWS FOR WRC-15

Agenda Item 1.4:  to consider possible new allocation to the amateur service on a secondary basis within the band 5 250-5 450 kHz in accordance with Resolution 649 (WRC-12)

BACKGROUND:  Incumbent services in the 5 250-5 450 kHz range include the fixed, mobile (except aeronautical mobile), and radiolocation services.  Experience has shown that amateur service operation is incompatible with HF radiolocation; thus, the 5 250-5 275 kHz range is not suitable to satisfy this agenda item.  Some administrations, including the United States, have permitted amateur service licensees privileges within the 5 275-5 450 kHz range under Radio Regulations No. 4.4, in some cases permitting operation on discrete channels, and in other cases permitting access to a frequency band.  These amateur operations are typically limited to lower power levels (i.e., 100 watts effective isotropic radiated power).  In some cases, these amateur operations are restricted to specific emission designators (i.e., 2K80J3E, 2K80J2D, 60HOJ2B and 150HA1A).

U.S. VIEW:  If ITU-R studies demonstrate compatibility with incumbent services, the United States will consider supporting a secondary allocation of up to 15 kHz to the amateur service within the 5 275-5 450 kHz range.  Contiguous spectrum is not a requirement for amateur operations in the band.  Sharing studies should consider non-contiguous, discreet allocations as well as listen-before-transmit protocols to provide additional protection for the primary services.

The allocation to the radiolocation service is in the band 5 250-5 275 kHz and comes into force on 1 January 2013.