| XXII MEETING OF PERMANENT CONSULTATIVE COMMITTEE II: RADIOCOMMUNICATIONS INCLUDING BROADCASTING March 17 to 21, 2013 Cartagena de Indias, Colombia |  | OEA/Ser.L/XVII.4.2 CCP.II-RADIO/doc. 3573/14 19 March, 2014 Original: English |  |
| --- | --- | --- | --- |
|  | AGENDA ITEM 1.11: PRELIMINARY VIEWS FOR WRC-15 |  |  |
|  | (Item on the Agenda: 3.1 (SGT-3)) |  |  |
|  | (Document submitted by the Coordinator) |  |  |

SGT 3 – Space Science and MSS issues

Coordinator: Mr. Tarcisio BAKAUS, BRAZIL (bakaust@anatel.gov.br)

Alternate Coordinator: Mr. Glenn FELDHAKE, UNITED STATES OF AMERICA (glenn.s.feldhake@nasa.gov)

Rapporteur Agenda Item: Mr. Glenn FELDHAKE, UNITED STATES OF AMERICA (glenn.s.feldhake@nasa.gov)

Alternate Rapporteur Agenda Item: Mr. Muya WACHIRA, CANADA (muya.wachira@ic.gc.ca)

Agenda Item 1.11:  to consider a primary allocation for the Earth exploration-satellite service (Earth-to-space) in the 7-8 GHz range, in accordance with Resolution 650 (WRC-12);

BACKGROUND: (Source: 3430-1-11r2_i)

The Earth exploration-satellite service (EESS) requires an additional Earth-to-space allocation in the frequency band 7 190-7 235 MHz because of congestion in the bands 2 025-2 110 MHz and 2 200-2 290 MHz, which currently support several hundred satellites, making coordination extremely difficult.

This Earth-to-space allocation, along with existing space-to-Earth allocations in the band 8 025-8 400 MHz, would also allow EESS satellites to employ a single transponder for both uplinks and downlinks, reducing design and launch costs.  Currently, no suitable Earth-to-space allocations are available for tracking, telemetry and control (TT&C) of EESS satellites at frequencies higher than the 2 025-2 110 MHz global allocation.  Additionally, the band 2 200-2 290 MHz can support payload data downlinks for only a few EESS satellites.  These factors require current EESS satellites to be equipped with two transponders: one operating near 2 GHz for TT&C and the other operating at the higher frequencies required for medium- and high-rate payload data downlinks, typically in the band 8 025-8 400 MHz.  With a suitable EESS Earth-to-space allocation near the 8 025-8 400 MHz band, a single transponder could accommodate both satellite control and payload data downlink requirements.

The band 7 145-7 235 MHz is currently allocated to the fixed, mobile and, space research (Earth-to-space) services on a primary basis, subject to the conditions on the use of the space research service in No. 5.460.  ITU-R preliminary studies suggest that sharing with existing services within 7 190-7 235 MHz may be feasible, while difficulties have been cited with regard to sharing in the 7 145-7 190 MHz band segment, where the use of the space research service is restricted by No. 5.460 to deep space.  Most of the remainder of the 7-8 GHz band is allocated to the fixed-satellite service (FSS) on a worldwide primary basis.  Additional preliminary studies suggest that sharing may be difficult between EESS and FSS in this frequency range.

- ISSUES:

- Accommodate the TT&C spectrum requirement of future space science missions.

- Protect existing services (FS, MS and SRS) from potential interference.

- Possible linkage with WRC-15 Agenda Items 1.9.1 and 1.9.2.

PRELIMINARY VIEWS:

B/CAN/USA

Support a new primary allocation for the Earth exploration-satellite service (Earth-to-space) to accommodate the TT&C spectrum requirement if the studies in accordance with Resolution 650 (WRC-12) prove compatibility with existing services.

CAN/USA

Supports studies within portions of the 7-8 GHz range with priority to the band 7 190-7 235 MHz.  Supports consideration of an allocation within other portions of the 7-8 GHz range only if EESS (Earth-to-space) in the 7 190-7 235 MHz band proves insufficient or is not feasible, noting that sharing may be difficult between EESS and existing globally allocated FSS in the 7-8 GHz range.

__________________________