| XXIII MEETING OF PERMANENT CONSULTATIVE COMMITTEE II: RADIOCOMMUNICATIONS March 17 to 21, 2014 Cartagena de Indias, Colombia |  | OEA/Ser.L/ XVII.4.2 CCP.II-RADIO/doc. 3573/14 20 March 2014 Original: English |  |
| --- | --- | --- | --- |
|  | AGENDA ITEM 1.2: PRELIMINARY VIEWS FOR WRC-15 |  |  |
|  | (Item on the Agenda: 3.1 (SGT1)) |  |  |
|  | (Document submitted by the Coordinator) |  |  |

SGT 1 – Mobile and Fixed Issues

Coordinator: Mr. Marco ESCALANTE, GUATEMALA (marco.escalante@ties.itu.int)

Alternate Coordinator: Mr. José COSTA, CANADA (jose.costa@ericsson.com)

Rapporteur Agenda Item: TBC

Alternate Rapporteur Agenda Item: Mr. Agostino LINHARES, BRAZIL (linhares@anatel.gov.br)

Agenda Item 1.2	to examine the results of ITU-R studies, in accordance with Resolution 232, on the use of the frequency band 694-790 MHz by the mobile, except aeronautical mobile, service in Region 1 and take the appropriate measures;

BACKGROUND:

WRC-07 identified portions of the 698-862 MHz band (698-806 MHz in Region 2 and some countries in Region 3, 790-862 MHz in Regions 1& 3) for IMT.  WRC-12 allocated the band 694-790 MHz in Region 1 to the mobile service on a primary basis and identified the band for IMT.  The allocation is effective immediately after WRC-15.  As well, the lower edge of the allocation is subject to refinement at WRC-15.

In Resolution 232 (WRC-12), the ITU-R is invited to study the spectrum requirement for the mobile service and for the broadcasting service in this frequency band, in order to determine possible options for the lower edge of the frequency band.  The ITU-R is also invited to study the channelling arrangements for the mobile service, adapted to the frequency band below 790 MHz, taking into account:

- The existing arrangements in Region 1 in the bands between 790 and 862 MHz and defined in the last version of Recommendation ITU-R M.1036, in order to ensure coexistence with the networks operated in the new allocation and the operational networks in the band 790-862 MHz,

- The desire for harmonization with arrangements across all Regions,

- Compatibility with other primary services to which the band is allocated, including in adjacent bands;

In addition, the ITU-R is invited to study coexistence between different channelling arrangements which have been implemented in Region 1 above 790 MHz, as well as the possibility of further harmonization.  The ITU-R is also invited to study the compatibility between the mobile service and other services currently allocated in the frequency band 694-790 MHz. Finally, the ITU-R is invited to study solutions for accommodating applications ancillary to broadcasting requirements.

ITU-R Joint Task Group (JTG) 4-5-6-7 is the responsible group for this agenda item.  The group is tasked with developing sharing studies and draft CPM text taking into consideration the results of studies from ITU-R Working Party (WP) 5D on the spectrum requirements for the mobile service, including suitable frequency ranges, as well as input from any concerned Working Parties (such as WP 6A) on technical and operational characteristics, spectrum requirements and performance objectives or protection requirements.  The channelling arrangement(s) for the mobile service in the frequency range 694-862 MHz is to be carried out in WP 5D.

The decision by World Radiocommunication Conference 2012 (WRC-12) to allocate the frequency band 694-790 MHz to the mobile, except aeronautical mobile, service in Region 1, subject to the provisions of Resolution 232 (via Footnote 5.312A), led to the adoption of Agenda Item 1.2 for WRC-15 to examine the results of ITU-R studies, in accordance with Resolution 232 and take the appropriate regulatory actions. Resolves 2 through 5 of Resolution 232 pertain to the details of the regulatory actions that must be addressed by WRC-15: they are likely to have limited impact to administrations outside of Region 1 (and also those who are not signatories to the GE06 agreement). However, the studies called for under invites ITU-R 1 through 4 of Resolution 232 could lead to the development of sharing and compatibility methodologies that may subsequently be suggested for application to possible sharing and compatibility studies undertaken in the UHF band for WRC-15 Agenda Item 1.1.

According to the Terms of Reference for  the Joint Task Group (JTG) 4-5-6-7, the results of its studies can be submitted to the relevant ITU-R Study Groups and these Study Groups can approve ITU-R Recommendations and/or Reports developed by the JTG 4-5-6-7, as appropriate . Since the JTG 4-5-6-7 is the responsible group for both WRC-15 Agenda Items 1.1 and 1.2, it is possible that some Administrations might exert pressure to utilize the same sharing and compatibility methodologies for the studies to be undertaken in support of both agenda items within the JTG.

- ISSUES:

The main concern is to ensure that the results of the studies undertaken by administrations regarding the 694-790 MHz band in Regions 1:

- do not lead to regulatory constraints that may adversely impact the interests of countries in Region 2 in the 470-698 MHz or 698-790 MHz frequency ranges;

- do not establish any precedence for Agenda Item 1.1 studies concerning Region 2, especially regarding the aspects of spectrum requirements or compatibility studies;

- while this is largely a Region 1 issue, harmonization of frequency use and channelling arrangements on a global basis would be beneficial for all Regions.

PRELIMINARY VIEWS:

CAN/USA

Are of the view that the studies undertaken by JTG 4-5-6-7 to address WRC-15 Agenda Items 1.1 and 1.2 are separate and distinct, even if bands of interest to both agenda items prove to be similar. Therefore, sharing and compatibility methodologies that may be utilized in possible sharing and compatibility studies undertaken for WRC-15 Agenda Item 1.2 will not a priori be agreed for application to studies under WRC-15 Agenda Item 1.1. Furthermore, there is no basis for any change to the Radio Regulations being addressed under WRC-15 Agenda Item 1.2 that pertain to, or otherwise impact, Region 2.

_______________________

CCP.II-RADIO/doc. 3430/13 rev.2