| XXIII MEETING OF PERMANENT CONSULTATIVE COMMITTEE II: RADIOCOMMUNICATIONS March 17 to 21, 2014 Cartagena de Indias, Colombia |  | OEA/Ser.L/XVII.4.2 CCP.II-RADIO/doc. 3573/14 18 March 2014 Original: English |  |
| --- | --- | --- | --- |
|  | AGENDA ITEM 1.9.1: PRELIMINARY views for WRC-15 |  |  |
|  | (Item on the Agenda: 3.1 (SGT4)) |  |  |
|  | (Document submitted by the Coordinator) |  |  |

SGT 4 – FSS and satellite regulatory issues

Coordinator:  Mr. Jerry CONNER, UNITED STATES OF AMERICA (jerry.conner@exelisinc.com)

Alternate Coordinator:  Ms. Chantal BEAUMIER, CANADA (chantal.beaumier@ic.gc.ca)

Mr. Ramiro ROBLEDO, MEXICO (rrobledo@cft.gob.mx)

Rapporteur Agenda Item: TBC

Alternate Rapporteur Agenda Item: TBC

Agenda item 1.9.1:   to consider, in accordance with Resolution 758 (WRC-12), possible new allocations to the fixed-satellite service in the frequency bands 7 150 – 7 250 MHz (space-to-Earth) and 8 400 – 8 500 MHz (Earth-to-space), subject to appropriate sharing conditions;

BACKGROUND:

The frequency bands 7 250-7 750 MHz (space-to-Earth) and 7 900-8 400 MHz (Earth-to-space) are allocated worldwide to the fixed-satellite service (FSS).  Some administrations report a shortfall of spectrum for their current and future FSS applications, and estimate an additional bandwidth requirement of up to 100 MHz for both uplink and downlink data transmission on FSS next-generation satellites.

The bands under study for the new FSS allocations are 7 150-7 250 MHz and 8 400-8 500 MHz.  Both bands are currently allocated to the space research service (SRS), fixed service, and mobile service.  The use of the bands 7 145-7 190 MHz and 8 400-8 450 MHz by the SRS is limited to deep space.  The SRS supports near Earth missions in the bands 7 190-7 235 MHz and 8 450 -8 500 MHz.  Currently, there are no other space services co-allocated with primary SRS (deep space) anywhere in the Radio Regulations.

In accordance with Resolution 758 (WRC-12), resolves 2, appropriate regulatory studies should be conducted to “...ensure that any new FSS allocation referred to in resolves 1 above is limited to FSS systems operated from a fixed known location in order to enable compatibility with systems of other services, taking into account that operational requirements in the bands 7 150-7 250 MHz (space-to-Earth) and 8 400-8 500 MHz (Earth-to-Space) do not encompass small VSAT-like FSS earth stations.”

ISSUES:

- Possible linkage with agenda item 1.11 (Resolution 650 (WRC-12), which is considering a new primary allocation for the Earth Exploration-Satellite Service (EESS) in the Earth-to-space direction in the 7-8 GHz frequency range.

PRELIMINARY VIEWS:

CAN/USA/MEX

If ITU-R studies demonstrate compatibility with incumbent services and if due consideration is given to a potential allocation to EESS under Agenda Item 1.11, these administrations will consider supporting allocations to the FSS in the bands 7 150 – 7 250 MHz and 8 400 – 8 500 MHz, or portions thereof, limited to FSS systems operated from a fixed, known location not encompassing small VSAT-like FSS earth stations.

__________________________

CCP.II-RADIO/doc. 3283/13 rev.3