| XXIV MEETING OF PERMANENT CONSULTATIVE COMMITTEE II: RADIOCOMMUNICATIONS September 29 to October 3, 2014 Mérida City, Yucatán, México |  | OEA/Ser.L/XVII.4.2 CCP.II-RADIO/doc. 3692/14 1 October 2014 Original: English |  |
| --- | --- | --- | --- |
|  | AGENDA ITEM 1.16: PRELIMINARY PROPOSAL FOR WRC-15 |  |  |
|  | (Item on the Agenda: 3.1 (SGT2)) |  |  |
|  | (Document submitted by the Coordinator) |  |  |

SGT 2 – Radiolocation, Amateur, Maritime, and Aeronautical issues

Coordinator: Mr. Jonathan WILLIAMS, UNITED STATES OF AMERICA (jwilliams@ntia.doc.gov)

Alternate Coordinator: Mr. Javier GARCIA, DOMINICAN REPUBLIC (jgarcia@indotel.gob.do)

Rapporteur Agenda Item: Mr. Bill KAUTZ, UNITED STATES OF AMERICA (William.d.Kautz@uscg.mil)

Alternate Rapporteur Agenda Item: Mr. Camilo ZAMORA, COLOMBIA (Camilo.zamora@ane.gov.co)

Agenda Item 1.16:  to consider regulatory provisions and spectrum allocations to enable possible new Automatic Identification System (AIS) technology applications and possible new applications to improve maritime radiocommunication in accordance with Resolution 360 [COM6/21](WRC-12)

Resolution 360 (WRC-2012): Consideration of regulatory provisions and spectrum allocations for enhanced Automatic Identification System technology applications and for enhanced maritime radiocommunication

Background Information:  This agenda item addresses regulatory provisions and spectrum allocations for use by maritime safety systems for ships and ports.

Since AIS 1 and AIS 2 are very close in frequency to channels 2078, 2019, 2079 and 2020, the use of these channels for radio communications by ships will block the ship’s AIS receiver, consequentially causing the ship’s AIS to be unable to update the location of other ships nearby, resulting in a navigation safety hazard and possible collision. This problem should be solved, not only to protect the AIS channels, but also to protect the additional channels that may be allocated to support AIS technology applications.

Proposals:

[SOURCE: 3574]

MOD	USA/1.16/1

APPENDIX 18 (Rev.WRC-12)

Table of transmitting frequencies in the VHF maritime mobile band

(See Article 52)

…

| Channel designator | Notes | Transmitting frequencies (MHz) |  | Inter-ship | Port operations and ship movement |  | Public corres-pondence |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | From ship stations | From coast stations |  | Single frequency | Two frequency |  |
| 60 | m) | 156.025 | 160.625 |  | x | x | x |
| 01 | m) | 156.050 | 160.650 |  | x | x | x |
| 61 | m) | 156.075 | 160.675 |  | x | x | x |
| 02 | m) | 156.100 | 160.700 |  | x | x | x |
| 62 | m) | 156.125 | 160.725 |  | x | x | x |
| 03 | m) | 156.150 | 160.750 |  | x | x | x |
| 63 | m) | 156.175 | 160.775 |  | x | x | x |
| 04 | m) | 156.200 | 160.800 |  | x | x | x |
| 64 | m) | 156.225 | 160.825 |  | x | x | x |
| 05 | m) | 156.250 | 160.850 |  | x | x | x |
| 65 | m) | 156.275 | 160.875 |  | x | x | x |
| 06 | f) | 156.300 |  | x |  |  |  |
| 2006 | r) | 160.900 | 160.900 |  |  |  |  |
| 66 | m) | 156.325 | 160.925 |  | x | x | x |
| 07 | m) | 156.350 | 160.950 |  | x | x | x |
| 67 | h) | 156.375 | 156.375 | x | x |  |  |
| 08 |  | 156.400 |  | x |  |  |  |
| 68 |  | 156.425 | 156.425 |  | x |  |  |
| 09 | i) | 156.450 | 156.450 | x | x |  |  |
| 69 |  | 156.475 | 156.475 | x | x |  |  |
| 10 | h), q) | 156.500 | 156.500 | x | x |  |  |
| 70 | f), j) | 156.525 | 156.525 | Digital selective calling for distress, safety and calling |  |  |  |
| 11 | q) | 156.550 | 156.550 |  | x |  |  |
| 71 |  | 156.575 | 156.575 |  | x |  |  |
| 12 |  | 156.600 | 156.600 |  | x |  |  |
| 72 | i) | 156.625 |  | x |  |  |  |
| 13 | k) | 156.650 | 156.650 | x | x |  |  |
| 73 | h), i) | 156.675 | 156.675 | x | x |  |  |
| 14 |  | 156.700 | 156.700 |  | x |  |  |
| 74 |  | 156.725 | 156.725 |  | x |  |  |
| 15 | g) | 156.750 | 156.750 | x | x |  |  |
| 75 | n), s) | 156.775 | 156.775 |  | x |  |  |
| 16 | f) | 156.800 | 156.800 | DISTRESS, SAFETY AND CALLING |  |  |  |
| 76 | n), s) | 156.825 | 156.825 |  | x |  |  |
| 17 | g) | 156.850 | 156.850 | x | x |  |  |
| 77 |  | 156.875 |  | x |  |  |  |
| 18 | m) | 156.900 | 161.500 |  | x | x | x |
| 78 | t), u), v) | 156.925 | 161.525 |  | x | x | x |
| 1078 |  | 156.925 | 156.925 |  | x |  |  |
| 2078 | tt) |  | 161.525 |  | x |  |  |
| 19 | t), u), v) | 156.950 | 161.550 |  | x | x | x |
| 1019 |  | 156.950 | 156.950 |  | x |  |  |
| 2019 | tt) |  | 161.550 |  | x |  |  |
| 79 | t), u), v) | 156.975 | 161.575 |  | x | x | x |
| 1079 |  | 156.975 | 156.975 |  | x |  |  |
| 2079 | tt) |  | 161.575 |  | x |  |  |
| 20 | t), u), v) | 157.000 | 161.600 |  | x | x | x |
| 1020 |  | 157.000 | 157.000 |  | x |  |  |
| 2020 | tt) |  | 161.600 |  | x |  |  |
| 80 | w), y) | 157.025 | 161.625 |  | x | x | x |
| 21 | w), y) | 157.050 | 161.650 |  | x | x | x |
| 81 | w), y) | 157.075 | 161.675 |  | x | x | x |
| 22 | w), y) | 157.100 | 161.700 |  | x | x | x |
| 82 | w), x), y) | 157.125 | 161.725 |  | x | x | x |
| 23 | w), x), y) | 157.150 | 161.750 |  | x | x | x |
| 83 | w), x), y) | 157.175 | 161.775 |  | x | x | x |
| 24 | w), ww), x), y) | 157.200 | 161.800 |  | x | x | x |
| 84 | w), ww), x), y) | 157.225 | 161.825 |  | x | x | x |
| 25 | w), ww), x), y) | 157.250 | 161.850 |  | x | x | x |
| 85 | w), ww), x), y) | 157.275 | 161.875 |  | x | x | x |
| 26 | w), ww), x), y) | 157.300 | 161.900 |  | x | x | x |
| 86 | w), ww), x), y) | 157.325 | 161.925 |  | x | x | x |
| 27 | z) | 157.350 | 161.950 |  |  | x | x |
| 87 | z) | 157.375 | 157.375 |  | x |  |  |
| 28 | z) | 157.400 | 162.000 |  |  | x | x |
| 88 | z) | 157.425 | 157.425 |  | x |  |  |
| AIS 1 | f), l), p) | 161.975 | 161.975 |  |  |  |  |
| AIS 2 | f), l), p) | 162.025 | 162.025 |  |  |  |  |

Notes referring to the Table

…

ADD	USA/1.16/2

tt)	Channels 2078, 2019, 2079 and 2020 are not available for transmitting from ships.

[SOURCE: 3643]

ARTICLE 5

Frequency allocations

Section IV – Table of Frequency Allocations 
(See No. 2.1)

MOD	CAN/1.16/1

148-223 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 156.8375-161.9375 FIXED MOBILE except aeronautical mobile | 156.8375-161.9375 FIXED MOBILE |  |
| 5.226 | 5.226 |  |
| 161.9375-161.9625 FIXED MOBILE except aeronautical mobile Maritime mobile-satellite (Earth-to-space) | 161.9375-161.9625 FIXED MOBILE Maritime mobile-satellite (Earth-to-space) |  |
| 5.226 ADD 5.226A | 5.226 ADD 5.226A |  |

148-223 MHz (end)

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 161.9625-161.9875 FIXED MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) 5.228F | 161.9625-161.9875 AERONAUTICAL MOBILE (OR) MARITIME MOBILE MOBILE-SATELITE (Earth-to-space) | 161.9625-161.9875 MARITIME MOBILE Aeronautical mobile (OR) 5.228E Mobile-satellite (Earth-to-space) 5.228F |
| 5.226 5.228A 5.228B | 5.228C 5.228D | 5.226 |
| 161.9875-162.0125 FIXED MOBILE except aeronautical mobile Maritime mobile-satellite (Earth-to-space) | 161.9875-162.0125 FIXED MOBILE Maritime mobile-satellite (Earth-to-space) |  |
| 5.226 ADD 5.226A 5.229 | 5.226 ADD 5.226A |  |

Reasons:	The above modifications of RR Article 5 identify a MMSS allocation uplink to allow satellite reception of the frequencies 161.950 MHz and 162.000 MHz assigned for application specific messages (ASM).

ADD	CAN/1.16/2

5.226A	The use of the frequency bands 161.9375-161.9625 MHz and 161.9875-162.0125 MHz by the maritime mobile-satellite (Earth-to-space) service is limited to the systems which operate in accordance with Appendix 18. (WRC-15)

Reasons:	The above new footnote will limit use of the maritime mobile-satellite (Earth-to-space) service to the designated ASM channels identified in Note z) of Appendix 18.

APPENDIX 18 (Rev.WRC-15)

Table of transmitting frequencies in the
VHF maritime mobile band

(See Article 52)

MOD	CAN/1.16/3

| Channel designator | Notes | Transmitting frequencies (MHz) |  | Inter-ship | Port operations and ship movement |  | Public corres-pondence |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | From ship stations | From coast stations |  | Single frequency | Two frequency |  |
| 15 | g) | 156.750 | 156.750 | x | x |  |  |
| 75 | n), s) | 156.775 | 156.775 |  | x |  |  |
| 16 | f) | 156.800 | 156.800 | DISTRESS, SAFETY AND CALLING |  |  |  |
| 76 | n), s) | 156.825 | 156.825 |  | x |  |  |
| 17 | g) | 156.850 | 156.850 | x | x |  |  |
| 77 |  | 156.875 |  | x |  |  |  |
| 18 | m) | 156.900 | 161.500 |  | x | x | x |
| 78 | t), u), v) | 156.925 | 161.525 |  | x | x | x |
| 1078 |  | 156.925 | 156.925 |  | x |  |  |
| 2078 |  | 161.525 | 161.525 |  | x |  |  |
| 19 | t), u), v) | 156.950 | 161.550 |  | x | x | x |
| 1019 |  | 156.950 | 156.950 |  | x |  |  |
| 2019 |  | 161.550 | 161.550 |  | x |  |  |
| 79 | t), u), v) | 156.975 | 161.575 |  | x | x | x |
| 1079 |  | 156.975 | 156.975 |  | x |  |  |
| 2079 |  | 161.575 | 161.575 |  | x |  |  |
| 20 | t), u), v) | 157.000 | 161.600 |  | x | x | x |
| 1020 |  | 157.000 | 157.000 |  | x |  |  |
| 2020 |  | 161.600 | 161.600 |  | x |  |  |
| 80 | w), y) | 157.025 | 161.625 |  | x | x | x |
| 21 | w), y) | 157.050 | 161.650 |  | x | x | x |
| 81 | w), y) | 157.075 | 161.675 |  | x | x | x |
| 22 | w), y) | 157.100 | 161.700 |  | x | x | x |
| 82 | w), x), y) | 157.125 | 161.725 |  | x | x | x |
| 23 | w), x), y) | 157.150 | 161.750 |  | x | x | x |
| 83 | w), x), y) | 157.175 | 161.775 |  | x | x | x |

| 24 | w), ww), x), y) | 157.200 | 161.800 |  | x | x | x |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 84 | w), ww), x), y) | 157.225 | 161.825 |  | x | x | x |
| 25 | w), ww), x), y) | 157.250 | 161.850 |  | x | x | x |
| 85 | w), ww), x), y) | 157.275 | 161.875 |  | x | x | x |
| 26 | w), ww), x), y) | 157.300 | 161.900 |  | x | x | x |
| 86 | w), ww), x), y) | 157.325 | 161.925 |  | x | x | x |
| 27 | z) | 157.350 | 161.950 |  |  | x | x |
| 1027 | dd) | 157.350 |  |  | x |  |  |
| 2027 | ddd), z) | 161.950 | 161.950 |  | x |  |  |
| 87 |  | 157.375 | 157.375 |  | x |  |  |
| 28 | z) | 157.400 | 162.000 |  |  | x | x |
| 1028 | dd) | 157.400 |  |  | x |  |  |
| 2028 | ddd), z) | 162.000 | 162.000 |  | x |  |  |
| 88 |  | 157.425 | 157.425 |  | x |  |  |
| AIS 1 | f), l), p) | 161.975 | 161.975 |  |  |  |  |
| AIS 2 | f), l), p) | 162.025 | 162.025 |  |  |  |  |

Reasons:

Introduction of channels in Appendix 18 for new AIS technology applications: ASM 1 (161.950) and ASM 2 (162.000) as non-navigation application specific messages.  With the designation of ASM1 and ASM2, note z) may be removed from channels 87 and 88 of Appendix 18.

Notes referring to the Table

NOC

Notes a) to s)

MOD	CAN/1.16/4

t)	In Regions 1 and 3, the existing duplex channels 78, 19, 79 and 20 can continue to be assigned. These channels may be operated as single-frequency channels, subject to coordination with affected administrations. However, existing duplex channel assignments may be preserved for coast stations and retained for vessels, subject to coordination with affected administrations.  Channels 2078, 2019, 2079 and 2020 are not available for transmitting from ships.  (WRC-15)

u)	In Region 2, these channels may be operated as single-frequency channels, subject to coordination with affected administrations.   Channels 2078, 2019, 2079 and 2020 are not available for transmitting from ships. (WRC-15)

v)	After 1 January 2017, in the Netherlands, these channels may continue to be operated as duplex frequency channels, subject to coordination with affected administrations.   Channels 2078, 2019, 2079 and 2020 are not available for transmitting from ships. (WRC-15)

Reasons:	The split of the channels 78, 19, 79, 20 and the use of the upper legs of these channels could cause interference to AIS equipment. Therefore it is proposed that channels 2078, 2019, 2079 and 2020 will not be available for transmitting from ships.

NOC

Note w and ww)

NOC

Notes x) and y)

MOD	CAN/1.16/5

z)	Until 1 January 2019, these channels may be used for possible testing of future AIS applications without causing harmful interference to, or claiming protection from, existing applications and stations operating in the fixed and mobile services.    -

From 1 January 2019, these channels are split into two simplex channels. The upper legs, 2027 and 2028 respectively designated as ASM 1 and ASM 2 are used for non-navigation ASM (application specific messages) as described in the most recent version of the Recommendation/Report ITU-R M.[VDES].  The channels 2027 and 2028 are also allocated to the maritime mobile-satellite service (Earth-to-space) for the reception of ASM messages  as described in the most recent version of the Recommendation/Report ITU-R M.[VDES] in which they are denominated respectively as SAT up1 and SAT up2.  (WRC-15)

Reasons:	Identification of two channels dedicated to ASM applications not necessary for the security of the navigation, in order to secure the integrity and reduce loading of the channels AIS1 and AIS2.

MOD	CAN/1.16/6

ADD

dd)	From 1 January 2019, these channels shall be used as simplex voice channels for single frequency port operations.

ddd)	From 1 January 2019, these channels may be used for application specific messages (ASM). These channels could be continuously used for simplex voice applications subject to coordinating with ASM application, and not claiming protection.   (WRC-15)

Reasons:	Implementation of transition date from duplex channels 27 and 28 to simplex channels 1027, 1028, 2027, 2028.

________________

_____________