| XXIV MEETING OF PERMANENT CONSULTATIVE COMMITTEE II: RADIOCOMMUNICATIONS September 29 to October 3, 2014 Mérida City, Yucatán, México |  | OEA/Ser.L/XVII.4.2 CCP.II-RADIO/doc. 3692/14 1 October 2014 Original: English |  |
| --- | --- | --- | --- |
|  | AGENDA ITEM 1.4: PRELIMINARY PROPOSAL FOR WRC-15 |  |  |
|  | (Item on the Agenda: 3.1 (SGT2)) |  |  |
|  | (Document submitted by the Coordinator) |  |  |

Agenda item 1.4:  to consider possible new allocation to the amateur service on a secondary basis within the band 5 250-5 450 kHz in accordance with Resolution 649 (WRC-12);

1.	INTRODUCTION

Resolution 649 (WRC-12):  Possible allocation to the amateur service on a secondary basis at around 5 300 kHz.

Canada submitted its preliminary proposal for WRC-15 agenda item 1.4 to the XXIII meeting of PCC.II in Cartagena de Indias, Colombia.  This was incorporated into the Rapporteur’s output document of the last meeting (CCP.II-RADIO/doc. 3574).

2.	BACKGROUND

As the responsible group for this agenda item, ITU-R Working Party 5A (WP 5A) has studied (and continues to study) sharing among the amateur services and other services currently allocated in and/or adjacent to the band 5 250 - 5 450 kHz.  Compatibility studies indicate that observance by the amateur service of listen-before-transmit protocols would not cause harmful interference to the primary fixed and mobile services in the range 5 275 kHz to 5 450 kHz.  A spectrum occupancy survey performed for the frequency range 5 250 kHz to 5 450 kHz in Canada over a period of one year has also identified available spectrum for amateur use.

As a result of these studies, Method A proposes an allocation to the amateur service, on a secondary basis, for one or more blocks of spectrum (not necessarily contiguous) in the range 5 275 kHz to 5 450 kHz.  Four sub-methods have been developed:

- Method A1 calling for an allocation to the amateur service, on a secondary basis in the frequency band 5 275 - 5 450 kHz,

- Method A2 calling for an allocation to the amateur service, on a secondary basis in the range 5 350 to 5 450 kHz,

- Method A3 calling for an allocation to the amateur service up to [xx] kHz, on a secondary basis with specific operating conditions, in the range 5 275 kHz to 5 450 kHz, and

- Method A4 calling for an allocation to the amateur service at several specific channels, on a secondary basis, in the range 5 275 kHz to 5 450 kHz.

3.	PROPOSAL

Canada submits further texts for the preliminary proposal pertaining to CITEL’s preparation to WRC-15 agenda item 1.4 as contained in Attachment 1.

Canada invites CITEL Administrations to review and contribute to, as appropriate, ITU-R and CITEL WRC preparatory activities with a view to formulating multi-country proposals to CPM15-2 and/or WRC-15 to address this issue.

Attachment 1

Source:  CCP.II-RADIO/doc. 3574/14

[Source: 3636]

Agenda item 1.4:  to consider possible new allocation to the amateur service on a secondary basis within the band 5 250-5 450 kHz in accordance with Resolution 649 (WRC-12);

Background:

Based on the recommendation of the 1978 CCIR Special Preparatory Meeting, WARC-79 accepted the principle that, like other high-frequency radio services, the amateur service should have access to a family of frequency bands such that communications can be maintained as propagation conditions change. The amateur radio service has access to allocations in the vicinity of 3 500 and 7 000 kHz; however, there are frequent occasions when ionospheric conditions render either or both of these allocations unsatisfactory for communications over the distances which amateur radio operators are frequently requested to cover in the course of facilitating emergency and disaster relief operations. These distances might be relatively short (less than 1 000 km) when providing direct support to first responders or relatively longer (greater than 1 000 km) when exchanging information, for example, with international organizations.

Therefore, to be equipped to provide communications at any time, including in times of emergency and disaster-relief, radio amateurs require access to frequencies in the vicinity of 5 300 kHz.

A number of administrations including, e.g. Bahrain, Bangladesh, Canada, the Czech Republic, Cayman Islands, the Dominican Republic, Finland, Ireland, Norway, Sweden, the United Kingdom, the United States and others have authorized, subject to various restrictions in addition to the  provisions of RR, Section II, Article 4.4, operation by amateur radio licensees within the 5 250-5 450 kHz frequency range.

Amateur service characteristics in the frequency range 5 250 to 5 450 kHz are similar to land mobile service with respect to antenna types, power, modulation, and transmission bandwidths.   Preliminary results indicate that the amateur service and land mobile service can coexist within the same spectrum range.

Experience has shown that amateur service operation is incompatible with HF radiolocation; thus, the 5 250 - 5 275 kHz range is not suitable to satisfy this agenda item.

Compatibility studies indicate that observance by the amateur service of listen-before-transmit protocols would not cause harmful interference to the primary fixed and mobile services in the 5 275 - 5 450 kHz range.

A spectrum occupancy survey performed for the frequency range 5 250 to 5 450 kHz in Canada over a period of one year identified available spectrum for amateur use.

Proposals:

Canada proposes to introduce and support the following implementation of Method A1: an allocation to the amateur service, on a secondary basis, in the frequency ranges 5 330 - 5 355 kHz and 5 405 - 5 430 kHz.

-

ARTICLE 5

Frequency allocations

Section IV – Table of Frequency Allocations 
(See No. 2.1)

MOD 	CAN/1.4/1

5 003-7 450 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| . . . |  |  |
| 5 060-5 250 FIXED Mobile except aeronautical mobile 5.133 |  |  |
| 5 250-5 275 FIXED MOBILE except aeronautical mobile Radiolocation 5.132A 5.133A | 5 250-5 275 FIXED MOBILE except aeronautical mobile RADIOLOCATION 5.132A | 5 250-5 275 FIXED MOBILE except aeronautical mobile Radiolocation 5.132A |
| 5 275-5 330 FIXED MOBILE except aeronautical mobile |  |  |
| 5 330-5 355 FIXED MOBILE except aeronautical mobile Amateur |  |  |
| 5 355-5 405 FIXED MOBILE except aeronautical mobile |  |  |
| 5 405-5 430 FIXED MOBILE except aeronautical mobile Amateur |  |  |
| 5 430-5 450 FIXED MOBILE except aeronautical mobile |  |  |
| 5 450-5 480 FIXED AERONAUTICAL MOBILE (OR) LAND MOBILE | 5 450-5 480 AERONAUTICAL MOBILE (R) | 5 450-5 480 FIXED AERONAUTICAL MOBILE (OR) LAND MOBILE |
| 5 480-5 680 AERONAUTICAL MOBILE (R) 5.111 5.115 |  |  |
| . . . |  |  |

Reason: To provide adequate spectrum for amateur service in the vicinity of 5 300 kHz. Compatibility studies have also indicated that observance by the amateur service of listen-before-transmit protocols would not cause harmful interference to the primary fixed and mobile services in the 5 275 - 5 450 kHz range; and a spectrum occupancy survey has identified available spectrum for amateur use in the frequency range 5 250 to 5 450 kHz.

SUP CAN/AI 1.4/2

RESOLUTION 649 (WRC-12)

Possible allocation to the amateur service on a secondary basis at around 5 300 kHz

Reason:  WRC-15 agenda item 1.4 has been satisfied.