| XXIV MEETING OF PERMANENT CONSULTATIVE COMMITTEE II: RADIOCOMMUNICATIONS September 29 to October 3, 2014 Mérida City, Yucatán, México |  | OEA/Ser.L/XVII.4.2 CCP.II-RADIO/ 01 October 2014 Original: English |  |
| --- | --- | --- | --- |
|  | AGENDA ITEM 7 (ANNEX 7): PRELIMINARY PROPOSAL FOR WRC-15 |  |  |
|  | (Item on the Agenda: 3.1 (SGT4)) |  |  |
|  | (Document submitted by the Coordinator) |  |  |

SGT 4 – FSS and satellite regulatory issues

Coordinator: Mr. Jerry CONNER, UNITED STATES OF AMERICA (jerry.conner@exelisinc.com)

Alternate Coordinator:  Ms. Chantal BEAUMIER, CANADA (chantal.beaumier@ic.gc.ca)

Mr. Ramiro ROBLEDO, MEXICO (ramiro robledo@ift.org.mx)

Rapporteur Agenda Item: Mr. Juan MASCIOTRA, ARGENTINA (jmasciotra@cnc.gov.ar)

Mr. Ramiro ROBLEDO, MEXICO (ramiro robledo@ift.org.mx)

Alternate Rapporteur Agenda Item: Mr. Scott KOTLER, UNITED STATES OF AMERICA 							(scott.kotler@lmco.com)

Ms. Chantal BEAUMIER, CANADA (chantal.beaumier@ic.gc.ca)

Source: doc.3616

ISSUE:    Appendix 30 Orbital Position Limitations

Agenda Item 7	to consider possible changes, and other options, in response to Resolution 86 (Rev. Marrakech, 2002) of the Plenipotentiary Conference, an advance publication, coordination, notification and recording procedures for frequency assignments pertaining to satellite networks, in accordance with Resolution 86 (Rev.WRC-07) to facilitate rational, efficient, and economical use of radio frequencies and any associated orbits, including the geostationary-satellite orbit;

Background Information:

Appendix 30 contains provisions for use of the BSS Plans and Regions 1 and 3 List, as well as for modifying the Plan (in the case of Region 2) or the List (in the case of Regions 1 and 3).  It is a self-contained Appendix, including provisions for modifying the Plan or List (Article 4), notifying Plan or List assignments and for coordinating other services in the frequency bands vis a vis the Plan and List (Articles 6 and 7).  Appendix 30 also contains detailed criteria for sharing between the Plan/List and other services.  In particular, the relevant provisions and associated technical criteria are:

–	Article 4 of Appendix 30  procedure for proposed modifications to the BSS Plan or List to coordinate with unplanned FSS or BSS.

–	Article 7 of Appendix 30  procedure for unplanned BSS or FSS networks to coordinate with BSS Plan or List assignments or previously filed modifications to 
the Plan or List.

–	Annex 1 to Appendix 30  criteria to determine if a proposed modification to the BSS Plan or List needs to coordinate with unplanned FSS or BSS networks.

−	The criteria here is a power-flux density (pfd) mask.

–	Annex 4 to Appendix 30  criteria to determine if an unplanned FSS or BSS network needs to coordinate with the BSS Plan or List assignments or previously filed modifications to the Plan or List.

−	The criteria here is a pfd mask.

–	Annex 6 to Appendix 30  summary of the assumptions used to develop the power flux density (pfd) levels contained in Annexes 1 and 4 to Appendix 30.

–	Annex 7 to Appendix 30  orbital position limitations on modifications to the BSS Plan or List; specifically applicable to Region 2 BSS in 12.2-12.7 GHz and to Region 1 BSS in 11.7-12.2 GHz. Annex 7 also contains associated e.i.r.p. limits for Region 1 BSS in a portion of the arc.

Annex 6 is particularly useful in understanding the derivation of the Annex 1 and 4 pfd limits, with respect to the earth stations considered and the desired T/T value.

The focus of the present review is on the orbital position limitations on modifications to the BSS Plans and List contained Annex 7 to Appendix 30.  These limitations were designed to facilitate sharing with the fixed-satellite service (FSS) in the shared part of the orbital arc between the Regions.  In the Ku band frequencies, the BSS allocations are not global, so, for example, 11.7-12.2 GHz is BSS in Region 1 and FSS in Region 2.  These orbital position limitations were maintained at WRC-2000 during the last Regions 1 and 3 planning conference, as during a planning conference, many new BSS slots could be adopted at once which could significantly limit the future access of FSS to the shared portion of the orbital arc.

Based on a review of the studies regarding the orbital separations allowed between BSS and FSS from the coordination triggers in Annexes 1 and 4 of Appendix 30 , representative BSS and FSS systems serving different regions can exist successfully with orbital separations as small as 0.5 and 2 degrees, depending on the carrier parameters and geographic discrimination assumed.  These small orbital separations suggest that additional measures, such as the orbital position limitations in Annex 7, are no longer needed outside of a planning conference.

Other factors to consider include:

- Both BSS and FSS may proceed at the same time to access the shared orbit resource outside of a planning conference.

- Special consideration may need to be given to operational systems implemented under the Annex 7 regime.

- Parity between the regions and services – BSS is subject to orbital position limitations while FSS in the same frequency bands are not.

Proposal:

MOD     PP/AI 7/1

Support:

United States of America

ANNEX  7     (Rev.WRC-03)

Orbital position limitations

Allowable portions of the orbital arc between 37.2°W and 10° E for new or modified 
assignments in the Regions 1 and 3 Plan and List

Nominal positions in the orbital arc between 37.2° W and 10° E at which the 
e.i.r.p. may exceed the limit of 56 dBW

The Region 2 Plan is based on the grouping of the space stations in nominal orbital positions of  0.2° from the centre of the cluster of satellites. Administrations may locate those satellites within a cluster at any orbital position within that cluster, provided they obtain the agreement of administrations having assignments to space stations in the same cluster. (See § 4.13.1 of Annex 3 to Appendix 30A.)

Reason:  Representative BSS and FSS systems serving different Regions can exist successfully with orbital separations as small as 0.5 and 2 degrees, depending on the carrier parameters and geographic discrimination assumed and therefore Paragraph A of Annex 7 to Appendix 30 is no longer necessary.

_____________