| XXII MEETING OF PERMANENT CONSULTATIVE COMMITTEE II: RADIOCOMMUNICATIONS INCLUDING BROADCASTING March 17 to 21, 2013 Cartagena de Indias, Colombia |  | OEA/Ser.L/XVII.4.2 CCP.II-RADIO/doc.3575/14 rev.1 19 March 2014 Original: English |  |
| --- | --- | --- | --- |
|  | AGENDA ITEM 1.13: Draft Inter-American PROPOSALS FOR WRC-15 |  |  |
|  | (Item on the Agenda: 3.1 (SGT3)) |  |  |
|  | (Document submitted by the Coordinator) |  |  |

SGT 3 – Space Science and MSS Issues

Coordinator: Mr. Tarcisio BAKAUS, BRAZIL (bakaust@anatel.gov.br)

Alternate Coordinator:  Mr. Glenn FELDHAKE, UNITED STATES OF AMERICA (glenn.s.feldhake@nasa.gov)

Rapporteur Agenda Item:  Mr. Edward R. JACOBS, UNITED STATES OF AMERICA

(Edward.r.jacobs@nasa.gov)

Alternate Rapporteur Agenda Item: TBC

Agenda Item 1.13:  to review No. 5.268 with a view to examining the possibility for increasing the 5 km distance limitation and allowing space research service (space-to-space) use for proximity operations by space vehicles communicating with an orbiting manned space vehicle, in accordance with Resolution 652  (WRC 12);

BACKGROUND:  (Source: 3432-1-13r1_i.doc)

WARC-92 allocated the band 410-420 MHz to the space research service (SRS) on a secondary basis for extra-vehicular activity (EVA) communications in the immediate vicinity of low earth orbit (LEO) manned space vehicles, and limited the use of the band by the SRS to EVA operation within 5 kilometers (km) of orbiting manned space vehicles.  WRC-97 upgraded the allocation to the SRS in the band 410-420 MHz to primary status and No. 5.268 specified a set of power flux-density (pfd) limits to ensure protection of the fixed and mobile services while retaining the 5 km distance limitation for EVA operation.

Resolution 652 (WRC-12), recognizing c, states that “power flux-density (pfd) limits contained in No. 5.268 ensure the protection of terrestrial stations operating in the fixed and mobile services independent of the distance from, or the source of, space-to-space communications in the SRS.”  Also, long-term space exploration objectives require new activities around a manned space station other than EVA, such as visiting vehicles for crew transportation/cargo re-supply and free-fly proximity vehicles for inspection and maintenance.  These vehicles need to initiate communication over distances greater than 5 km to ensure proper vehicle positioning, data exchange and system monitoring.  ITU-R sharing studies within Working Party 7B demonstrate that communication links for a variety of space vehicles other than EVA can meet the pfd limits in No. 5.268 for distances beyond 5 km by using different modulation, spreading technologies, and power control schemes (7B/88 Annex 1, Preliminary Draft New Report ITU-R SA.[Proximity operations] - “Sharing conditions between space research service proximity operations links and fixed and mobile service links in the 410-420 MHz band).

Therefore, it is necessary to modify No. 5.268 to remove both the 5 km distance limitation and restriction to EVA operation while maintaining the pfd limits.  Removal of these two restrictions will allow for greater flexibility in using the band 410-420 MHz for space research activities while maintaining protection of the terrestrial services.

PROPOSALS:

MOD 		DIAP/1.13/1

Support:

Brazil (Federative Republic of), Canada, United States of America, Mexico, Uruguay (Eastern Republic of)

[Antigua and Barbuda], [Argentine Republic], [Bahamas (Commonwealth of the)], [Barbados], [Belize], [Bolivia (Plurinational State of)], [Chile], [Colombia (Republic of)], [Costa Rica], [Dominica (Commonwealth of)], [Dominican Republic], [Ecuador], [El Salvador (Republic of)], [Grenada], [Guatemala (Republic of)], [Guyana], [Haiti (Republic of)], [Honduras (Republic of)], [Jamaica], [], [Nicaragua], [Panama (Republic of)], [Paraguay (Republic of)], [Peru], [Saint Lucia], [Saint Vincent and the Grenadines], [Saint Kitts and Nevis (Federation of)], [Suriname (Republic of)], [Trinidad and Tobago], [Venezuela (Bolivarian Republic of)]

5.268	Use of the band 410-420 MHz by the space research service is limited to space-to-space communications with  an orbiting, manned space vehicle. The power flux-density at the surface of the Earth produced by emissions from stations of the space research service (space-to-space) in the band 410-420 MHz shall not exceed –153 dB(W/m2) for 0    5, -153  0.077 ( – 5) dB(W/m2) for 5    70 and –148 dB(W/m2) for 70    , where  is the angle of arrival of the radio-frequency wave and the reference bandwidth is 4 kHz. In this frequency band the space research (space-to-space) service shall not claim protection from, nor constrain the use and development of, stations of the fixed and mobile services.   No. 4.10 does not apply.   (WRC-15)

Reasons:  Modify No. 5.268 to remove both the 5 km distance limitation and restriction to EVA operation while maintaining the pfd limits to protect the terrestrial services.

SUP		DIAP/1.13/2

Support:

Brazil (Federative Republic of), Canada, United States of America, Mexico, Uruguay (Eastern Republic of)

[Antigua and Barbuda], [Argentine Republic], [Bahamas (Commonwealth of the)], [Barbados], [Belize], [Bolivia (Plurinational State of)], [Chile], [Colombia (Republic of)], [Costa Rica], [Dominica (Commonwealth of)], [Dominican Republic], [Ecuador], [El Salvador (Republic of)], [Grenada], [Guatemala (Republic of)], [Guyana], [Haiti (Republic of)], [Honduras (Republic of)], [Jamaica], [], [Nicaragua], [Panama (Republic of)], [Paraguay (Republic of)], [Peru], [Saint Lucia], [Saint Vincent and the Grenadines], [Saint Kitts and Nevis (Federation of)], [Suriname (Republic of)], [Trinidad and Tobago], [Venezuela (Bolivarian Republic of)]

RESOLUTION 652 (WRC-12)

Use of the band 410-420 MHz by the space research service (space-to-space)

Reasons: ITU-R Working Party 7B completed required studies and this resolution is no longer needed.