LTE (FDD) Transmitter Characteristics

| Parameter | Base Station |  |  |  | Mobile/Portable Station |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Emission Bandwidth (MHz) | 1.4, 3, 5, 10, 15 and 20 MHz With signal bandwidths of 1.08, 2.7, 4.5, 9, 13.5 and 18 MHz |  |  |  | 1.4, 3, 5, 10, 15 and 20 MHz With signal bandwidths of 1.08, 2.7, 4.5, 9, 13.5 and 18 MHz |  |  |
| Power (EIRP) (dBm)/antenna | 61 (up to 5 MHz channel) 64 (> 10 MHz channel) (power control range of 20 dB) |  |  |  | 23 (power control range of 63 dB) |  |  |
| Emission Spectrum [Emission limit (dBm) as a Function of Frequency Offset of measurement filter center frequency (ΔF) from channel edge (MHz)] | Limit (dBm) | Meas BW | ΔF (MHz) |  | Limit (dBm) | Meas BW | ΔF (MHz) |
|  | 5 MHz Channel (BS) |  |  |  | 5 MHz Channel (UE) |  |  |
|  | -15 | 30 kHz | 0≤ΔF<1 |  | -15 | 30 kHz |  0-1 |
|  | -13 | 1 MHz | 1≤ΔF |  | -13 | 1 MHz |  1-2.5 |
|  |  |  |  |  | -13 | 1 MHz |  2.5-2.8 |
|  |  |  |  |  | -13 | 1 MHz |  2.8-5 |
|  |  |  |  |  | -13 | 1 MHz |  5-6 |
|  |  |  |  |  | -25 | 1 MHz |  6-10 |
|  |  |  |  |  | -30 | 1 MHz |  above 10 |
|  | 10 MHz Channel (BS) |  |  |  | 10 MHz Channel (UE) |  |  |
|  | -13 | 100 kHz |  | 0≤ΔF<1 | -18 | 30 kHz |  0-1 |
|  | -13 | 1 MHz |  | 1≤ΔF | -13 | 1 MHz |  1-2.5 |
|  |  |  |  |  | -13 | 1 MHz |  2.5-2.8 |
|  |  |  |  |  | -13 | 1 MHz |  2.8-5 |
|  |  |  |  |  | -13 | 1 MHz |  5-6 |
|  |  |  |  |  | -13 | 1 MHz |  6-10 |
|  |  |  |  |  | -25 | 1 MHz |  10-15 |
|  |  |  |  |  | -30 | 1 MHz |  above 15 |
|  | 15 MHz Channel (BS) |  |  |  | 15 MHz Channel (UE) |  |  |
|  | -15 | 100 kHz |  | 0≤ΔF<1 | -20 | 30 kHz |  0-1 |
|  | -13 | 1 MHz |  | 1≤ΔF | -13 | 1 MHz |  1-2.5 |
|  |  |  |  |  | -13 | 1 MHz |  2.5-2.8 |
|  |  |  |  |  | -13 | 1 MHz |  2.8-5 |
|  |  |  |  |  | -13 | 1 MHz |  5-6 |
|  |  |  |  |  | -13 | 1 MHz |  6-10 |
|  |  |  |  |  | -13 | 1 MHz |  10-15 |
|  |  |  |  |  | -25 | 1 MHz |  15-20 |
|  |  |  |  |  | -30 | 1 MHz |  above 20 |
|  | 20 MHz Channel (BS) |  |  |  | 20 MHz Channel (UE) |  |  |
|  | -16 | 100 kHz |  | 0≤ΔF<1 | -21 | 30 kHz |  0-1 |
|  | -13 | 1 MHz |  | 1≤ΔF | -13 | 1 MHz |  1-2.5 |
|  |  |  |  |  | -13 | 1 MHz |  2.5-2.8 |
|  |  |  |  |  | -13 | 1 MHz |  2.8-5 |
|  |  |  |  |  | -13 | 1 MHz |  5-6 |
|  |  |  |  |  | -13 | 1 MHz |  6-10 |
|  |  |  |  |  | -13 | 1 MHz |  10-15 |
|  |  |  |  |  | -13 | 1 MHz |  15-20 |
|  |  |  |  |  | -25 | 1 MHz |  20-25 |
|  |  |  |  |  | -30 | 1 MHz |  above 25 |
| Antenna Gain (Mainbeam) (dBi) | 18 |  |  |  | 0 |  |  |
| Azimuth Off-Axis Antenna Pattern (dBi as a function of off-axis angle in degrees) | Modified ITU-R Recommendation F.1336-2 |  |  |  | Omni-directional |  |  |
| Elevation Off-Axis Antenna Pattern (dBi as a function of off-axis angle in degrees) | ITU-R Recommendation F.1336-2 |  |  |  | Omni-directional |  |  |
| Antenna Height (meters)1 | 30 (Urban/Suburban) 15 to 60 (Rural) |  |  |  | 1.5 to 10 |  |  |
| Antenna Polarization | Linear |  |  |  | Linear |  |  |
| Antenna Azimuth 3 dB Beamwidth (degrees)3 | 70 |  |  |  | 360 |  |  |
| Antenna Down Tilt Angle (degrees) | 3 |  |  |  | 0 |  |  |
| Cable, Insertion, or Other Losses (dB) | 2 |  |  |  | 0 |  |  |
| Note 1: For single entry analysis, the maximum antenna height of 60 meters for base stations and 1.5 meters for mobile/portable stations will be used. For aggregate analysis antenna heights will be varied between the minimum and maximum values shown in the table. Note 3: A base station typically has three sectors each 120 degrees wide. |  |  |  |  |  |  |  |

LTE (FDD) Receiver Characteristics

| Parameter | Base Station |  | Mobile/Portable Station |  |  |
| --- | --- | --- | --- | --- | --- |
| Receiver Channel Bandwidth (MHz) | 1.4, 3, 5, 10, 15 and 20 With signal bandwidths of 1.08, 2.7, 4.5, 9, 13.5 and 18 MHz |  | 1.4, 3, 5, 10, 15 and 20 With signal bandwidths of 1.08, 2.7, 4.5, 9, 13.5 and 18 MHz |  |  |
| Adjacent Channel Selectivity (ACS) | Channel BW Wide Area BS | Wide Area BS Wanted Signal Mean Power (dBm) | Channel BW |  | ACS (dB) |
|  | 1.4 MHz 3 MHz 5 MHz 10 MHz 15 MHz 20 MHz Reference TS 36.104 Table 7.5.1-3 | -95.8 (PREFSENS + 11dB) -95.0 (PREFSENS + 8dB) -95.5 (PREFSENS + 6dB) -95.5 (PREFSENS + 6dB) -95.5 (PREFSENS + 6dB) -95.5 PREFSENS + 6dB Interfering signal mean power: -52 dBm | 1.4 MHz 3 MHz 5 MHz 10 MHz 15 MHz 20 MHz Reference TS 36.101 Table 7.5.1-1 |  | 33 33 33 33 30 27 |
|  | Channel BW Local Area BS | Local Area BS Wanted Signal Mean Power (dBm) |  |  |  |
|  | 1.4 MHz 3 MHz 5 MHz 10 MHz 15 MHz 20 MHz Reference TS 36.104 Table 7.5.1-4 | -87.8 (PREFSENS + 11dB) -87.0 (PREFSENS + 8dB) -87.5 (PREFSENS + 6dB) -87.5 (PREFSENS + 6dB) -87.5 (PREFSENS + 6dB) -87.5 (PREFSENS + 6dB) Interfering signal mean power: -44 dBm |  |  |  |
| Noise Figure (dB) | 5 |  | 9 |  |  |
| Reference Sensitivity (dBm) PREFSENS for Wide Area BS | 1.4 MHz 3 MHz 5 MHz 10 MHz 15 MHz 20 MHz | -106.8 -103.0 -101.5 -101.5 -101.5 -101.5 | 1.4 MHz 3 MHz 5 MHz 10 MHz 15 MHz 20 MHz | -101.7 -98.7 -97 -94 -92.2 -91 |  |
| Reference Sensitivity (dBm) PREFSENS for Local Area BS | 1.4 MHz 3 MHz 5 MHz 10 MHz 15 MHz 20 MHz | -98.8 -95.0 -93.5 -93.5 -93.5 -93.5 |  |  |  |
| Antenna Gain (Mainbeam) (dBi) | 18 |  | 0 |  |  |
| Azimuth Off-Axis Antenna Pattern (dBi as a function of off-axis angle in degrees) | Modified ITU-R Recommendation F.1336-2 |  | Omni-directional |  |  |
| Elevation Off-Axis Antenna Pattern (dBi as a function of off-axis angle in degrees) | ITU-R Recommendation F.1336-2 |  | Omni-directional |  |  |
| Antenna Polarization | Linear |  | Linear |  |  |
| Antenna Height (meters)1 | 30 (Urban/Suburban) 15 to 60(Rural) |  | 1.5 to 10 |  |  |
| Antenna Azimuth 3 dB Beamwidth (degrees)2 | 70 |  | 360 |  |  |
| Antenna Down Tilt Angle (degrees) | 3 |  | 0 |  |  |
| Cable, Insertion, or Other Losses (dB) | 2 |  | 0 |  |  |
| Note 1: For single entry analysis the maximum antenna height of 60 meters for base stations and 1.5 meters for mobile/portable stations will be used. For aggregate analysis antenna heights will be varied between the minimum and maximum values shown in the table. Note 2: A base station typically has three sectors each 120 degrees wide. |  |  |  |  |  |

See 3GPP TS 36.104, §5.6.

See 3GPP TS 36.101, §5.6.

See 3GPP TS 36.101, §6.6.2.3.2 at Table 6.6.2.3.2-1 for E-UTRA channel Measurement bandwidth.

See 3GPP TS 36.104, §6.3.2.1, 20 dB is for a 20 MHz channel bandwidth, dynamic power range will be reduced as channel bandwidth is decreased.

See 3GPP TS 36.101, §6.2.2.

See 3GPP TS 36.101, §6.3.2 for minimum transmit power of -40 dBm, with maximum transmit power of 23 dBm the power control range is 63 dB.

Values are only valid if the operating frequency is above 1 GHz.  See 3GPP TS 36.104, §6.6.3.2.1 for band class 4, Table 6.6.3.2.1-6 and 6.6.3.3-2.

See 3GPP TS 36.104 Tables 6.6.3.2.1-6 and 6.6.3.3-2 for frequency offset of measurement filter center frequency.

See 3GPP TS 36.101.  For 5 MHz channel at frequencies above 10 MHz offset the spurious emission levels apply from §6.6.4.1.2, for 10 MHz channels at frequencies above 15 MHz the spurious emission levels apply from §6.6.4.1.2.

This interfering signal mean power is for a wanted signal mean power at P_REFSENS + xdB (where x=6dB for 3-20MHz channels and 11dB for 1.4MHz channel). One way to interpret this spec is that this is the maximum interference level for xdB desense criterion. For instance, if 1dB desense is used in the coexistence studies, a conversion can be done to adjust for the lower desense criterion. For example, if adjacent channel selectivity is specified as -52dBm and wanted signal mean power is P_REFSENS + 6dB, the level can be adjusted by 11dB for the smaller sensitivity degradation allowed giving -52-11= -63dBm:

•    6 dB desense: maximum interference = Noise floor + 4.74 dB

•    1 dB desense: maximum interference = Noise floor  - 5.87 dB

Same as in endnote x, interfering signal mean power can be adjusted for 1dB desense if this criterion is used in the coexistence studies. For example, in the case of wanted signal mean power at P_REFSENS + 6dB, the level can be adjusted by 11dB for the smaller sensitivity degradation allowed giving -44-11=-55dBm.

See 3GPP TS 36.104, §7.2.  PREFSENS is the power level of a single instance of the reference measurement channel.  This requirement shall be met for each consecutive application of a single instance of FRC A1-3 mapped to disjoint frequency ranges with a width of 25 resource blocks each.